NATIONAL UNIVERSITY OF SINGAPORE

Department of Electrical and Computer Engineering

EE 5103 Kalman Filter

A0260045R Miao Chenxin

- Part 1

Given that the true state position xk of first order

xk+1=xk+w(k)

yk=xk+v(k)

Where the gaussian noise factor w(k) and v(k) have

σw=0.1  R1=0.01

σv=1           R2=1

And the initial value x0= 5, x0−1=0, P0−1=10^6. A Kalman model is designed based on this model.

The graphs and calculation can be shown below when k = 0,1…. 10000.

Graph 1: xk(solid line), x(k|k)(dotted line)

Graph 2: Pk|k

Graph 3: Kfk

Bias = -0.0033, Var = 0.0983

From graph 1 we can see that xk and x(k|k) do not has much difference in value, which can also be shown in bias result, which is – 0.0033. The estimation value is similar to true value with low error and variance, meaning that the Kalman filter estimation is accurate and has good performance in single value estimation. For graph 2, Pk|k is 1 when N =0, then it drops down quickly to and rapidly approaches to 0.0951. For graph 3, Kfk= 1 when N = 0 and approaches to 0.0951 quickly too. For variance = E{xk− xkkxk− xkkT}=0.0983, and when n >1 Pk|k = E{xk− xkkxk− xkkT} in theory. However, in estimation part the result we got is quite similar but not in the same value, it may be due to that the noise added is random.

- Part 2

Given that the true state position xk of second order.

[x1k+1 x2k+1 ] = 1T01 [x1k x2k ] + [0.5 T2T]w(k)

yk=[1  0][x1k x2k ]+v(k)

A= 1T01

B = [0.5 T2T]

C = [1  0]

Where the gaussian noise factor w(k) and v(k) have

σw=0.1  R1=0.01E{[0.5 T2wkTwk][0.5 T2wk  Twk]}

σv=1           R2=1

And T = 1, the initial value x10= 0, x20= 30. The initial setting is:

x10−1= x2(0|−1)= =0, P110|−1=P220|−1=10^6

A Kalman model is designed based on this model. The graphs and calculation can be shown below when k = 0,1…. 10000.

Graph 4: x1(k), x1(k|k)

Graph 5: x2(k), x2(k|k)

Graph 6: P11(k)

Graph 7: P22(k)

Graph 8: Kf1(k)

Graph 9: Kf2(k)

Bias_1 = -0.0210

Bias_2 =-0.0074

Var_1 = 0.3637

Var_2 = 0.1292

From graph 4 and graph 5 we can see that xk and x(k|k) do not has much difference in value, because x1k andx1kk, x2k and x2(k|k) both have low bias, which is -0.02 and -0.0074 respectively. The estimation value is similar to true value, meaning that the Kalman filter estimation is accurate and has good performance in second order value estimation. For graph 6, graph 7. P11k= 1 P22k=1 when n = 0, then they drop to 0.36 and 0.04 respectively at steady state. For graph 8 and graph 9, Kf1k=0.36 and Kf2k=0.08 at steady state. In theory, when at steady state, P11k= E{(x1(k)- x1(k|k))^2} = bias_1, P12k = E{(x1(k)- x1(k|k))(x2(k)-x2(k|k))} = P13k, P22k = E{(x2(k)- x2(k|k))^2} = bias_2, which can be confirmed by the estimation result.

- Part 3

This question can also be seen as a second order system since the position xk has dimension y and z in the figure, where x1k represents y axis and x2k represents z axis. The state model can be written as:

[xyk+1 xzk+1 ] = cospi/12−sinpi/12sin pi/12cospi/12 [xyk xzk ] + [1 1]w(k)

yk=[1  0]xyk+a(k)

zk=[0  1]xzk+b(k)

Where A = cospi/12−sinpi/12sin pi/12cospi/12, c1 = 1  0, c2 = [0  1], wk,ak, b(k) are gaussian noises variable and σw= σy=σz=1, therefore R = Ry = Rz =1. The initial point we set is the rightest point of the circle, which is (10,0) meaning that x10 = 10 and x20=0.

The initial setting is xy0−1=10cos12pi 10sin12pi=xz(0|−1),Py110|−1=Py220|−1=Pz110|−1=Pz220|−1=10^6. For both axis y and z, the Kalman filter is applied. The graphs and calculation can be shown below when k = 0,1…. 24.

Bias_y =-0.4849

Bias_z =-0.2431

Var_y =1.3291

Var_z =0.8633

The estimation bias and variance calculated are relatively low, which means the estimated point is not so far from the original positions as can be seen from the first graph. Kalman filter has a relatively good performance in position estimation of y and z axis. However, it still has bias and at first the deviation is relatively large, it may be due to the noise selected is large. For further improvement, the velocity steady state model may be added to have provide more information and have better modelling rather than single position estimation model.